NoteA1016B

JSE Router Replacement

Customer Checklist

| Created by: | J Smith |
| --- | --- |
| Approved by: |  |
| Date: | 8/10/2008 |
| Version | 0.1 |

# 1. Broker Responsibility

- Ensure they are able to log on to JSE systems they subscribe to once they have been cutover to the new routers

- Confirm that they have applied the necessary Routing / IP changes to their routers

- Confirm they have applied IP Changes to their Applications

- Ensure adequate backups are taken of desktop applications and router/switch configurations to facilitate roll-back, if required

- Ensure availability of technically skilled network and application resources, including 3rd party provider support (if required), to participate in the prep discussion leading up to the scheduled migration as well as on the cut-over day.  This is a huge effort and the need to repeat these activities due to the lack of adequate resources is costly.

- If a change is required at the user desktop / application level, testing of every workstation is recommended E.g. TALX ASP, Nutron

- Users to conduct impact analysis of their own environments with info included in the JSE Migration Checklist

- Broker to send JSE network team a copy of their Network Diagram

- Broker to confirm if they have to apply firewall changes in their environment

# 2.  Router Configurations

The following needs to be configured on your routers to allow you to successfully connect to the new 7600 Routers at the JSE:

| New Router Configs | Comment |
| --- | --- |
| Enable “pim sparse-mode” on the Serial and Ethernet interfaces | This enables multicast receiving and sending on an interface. Multicast will not flow if this is not enabled on the incoming and outgoing interfaces |
| Configure “join groups” on Ethernet interfaces | The join-group or static-group commands tell the interface of a router which multicast groups to listen for and forward. |
| Add a route on the serial interface that points to the new IP Range “196.216.152.0” | The JSE services are currently available on a 196.26.125.x range that we lease from Internet Solutions. The JSE purchased their own ranges and we are making the services available on the new 196.216.152.x range. For example SII is currently on 196.26.125.220 and the new NAT is 196.216.152.220 |
| Configure a RP Statement “ip pim rp-address 196.216.152.105 override” | This tells the router that the Rendezvous Point for multicast is at the JSE on IP address 196.216.152.105. The override keyword tells the router that this IP will always be the RP and it should not try to find other RP’s |
| Must have “IP mroute-cache” on all interfaces and Vlan’s where they receive multicast messages | This tells the router to process multicast from the router cache and not through the CPU. This takes quite a bit of load from the device and can also be applied to the interfaces and VLAN’s of the member’s switches. |
| Use 'ip igmp static-group' instead of 'ip igmp join-group' | This will reduce overall CPU utilization and improve multicast forwarding on the routers |
| Apply 'no ip pim dm-fallback' on all routers | This is an extra insurance against loss of RP information that will result in dense-mode behavior |
| Set 'ip pim query-interval 1' on last-hop routers | This will allow for faster DR (Designated Router) convergence during failure. Default PIM neighbor/DR convergence can be anywhere between 30-105 seconds. With the query-interval set to 1, the time to detect the loss/addition of a new PIM neighbor is 1-3 seconds. |
| Set 'ip pim dr-priority 100' on last-hop DR routers (leave non-DR routers alone). | Typically, you are trying to ensure the PIM DR is the same router as the HSRP active router for the given segment |

Note: All new communication providers like Neotel will terminate directly onto the new JSE Router Infrastructure.

Members can only connect with Static Routes to the JSE Network Infrastructure (See explanation below) – The option of load balancing will be allowed only after the broker has successfully migrated to the new routers.

Static Routes

If the member has more than one line they must choose which traffic ranges they want to route thru which line. The JSE will then add static routes with higher costs for the Backup Line.

Below is a sample config:

interface Serial0

description New Router 104

ip address 192.168.168.2 255.255.255.252

ip directed-broadcast

ip pim sparse-mode

encapsulation ppp

no fair-queue

clockrate 2000000

!

interface FastEthernet0

description Link_to_Test_LAN

ip address 99.99.98.1 255.255.255.248

ip directed-broadcast

ip pim sparse-mode

ip igmp static-group 239.5.5.10

ip igmp static-group 239.5.5.1

ip igmp static-group 239.5.5.2

ip igmp static-group 239.5.5.3

ip igmp static-group 239.5.5.11

ip igmp static-group 239.5.5.20

ip igmp static-group 239.5.5.21

ip igmp static-group 239.5.5.22

ip igmp static-group 239.5.5.23

ip igmp static-group 239.5.5.24

ip igmp static-group 239.5.5.25

ip igmp static-group 239.5.5.50

ip igmp static-group 239.5.5.51

ip igmp static-group 239.5.5.81

ip igmp static-group 239.5.5.82

ip igmp static-group 239.5.5.83

ip igmp static-group 239.5.5.84

ip igmp static-group 239.1.1.10

ip igmp static-group 239.1.1.11

ip cgmp

speed 100

full-duplex

!

router eigrp 100

network 99.99.98.0 0.0.0.7

network 192.168.168.0 0.0.0.3

no auto-summary

no eigrp log-neighbor-changes

!

ip classless

ip classless

ip route 196.216.152.0 255.255.254.0 192.168.168.1

ip route 196.26.120.0 255.255.254.0 192.168.168.1

ip pim rp-address 196.216.152.105 override

# 3. Router Replacement – Service Impact Summary

|  | Member Responsibility |  |  |  |  |  | JSE Responsibility |  |  | Test Proving | Test Method |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Service Subscriber Y / N | Application or Desktop FE |  | PRIMARY Router / Switch | Alternate Router(s) / Switch(es) e.g. Backup, DRP |  | JSE Router Config change | Application |  | PASS / FAIL |  |
| Equity Market |  |  |  |  |  |  |  |  |  |  |  |
| Production (H2H Clients) |  |  |  |  |  |  |  |  |  |  |  |
| JSE TradElect - SII |  | Connect to New IP Addr Same Port |  | New route per interface to new IP Range | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | - Customer to logon to SII - Customer to submit a trade report via their TRD USAP |
| Infowiz - IRI |  | Connect to New IP Addr Same Port |  | New route per interface to new IP Range | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | - Customer to logon to IRI - Customer to re-request messages across all their BDG’s - Customers logon to OOBD |
| InfoWiz - MultiCast Data |  | - |  | New MC Config changes per interface | New MC Config changes per interface |  | Apply new interface Config changes |  |  |  | - Confirm MC data flowing per BDG |
| Customer Development Service (CDS) |  |  |  |  |  |  |  |  |  |  |  |
| JSE TradElect - SII |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Infowiz - IRI |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| InfoWiz - MultiCast Data |  | - | New MC Config changes per interface |  | New MC Config changes per interface |  | Apply new interface Config changes |  |  |  | Confirm MC data flowing per BDG |
| Conformance |  |  |  |  |  |  |  |  |  |  |  |
| JSE TradElect - SII |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Infowiz - IRI |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| InfoWiz - MultiCast Data |  | - | New MC Config changes per interface |  | New MC Config changes per interface |  | Apply new interface Config changes |  |  |  | Confirm MC data flowing per BDG |
| TALX ASP Customers |  |  |  |  |  |  |  |  |  |  |  |
| Talx - ASP |  | Install new GLFlux File provided by the JSE | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes | Generate new GLFlux files |  |  | Login test sufficient |
| Equity Derivatives Market |  |  |  |  |  |  |  |  |  |  |  |
| Equity Derivatives Production |  |  |  |  |  |  |  |  |  |  |  |
| Equity Derivatives - Nutron |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Click Once - Auto FEUpgrade |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test Manual FE download |
| InfoMax EOD Files (ED product) |  | - | - |  | - |  | Ensure interface included in new Config file |  |  |  | Test File download |
| ISDN Dial-Up (Modem) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| ISDN Dial-Up (Router) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Equity Derivatives Test Environment |  |  |  |  |  |  |  |  |  |  |  |
| Equity Derivatives - Nutron |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Click Once - Auto FE Upgrade |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test Manual FE download |
| InfoMax EOD Files (ED product) |  | - | - |  | - |  | Ensure interface included in new Config file |  |  |  | Test File download |
| ISDN Dial-Up (Modem) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| ISDN Dial-Up (Router) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Agricultural Derivatives Market |  |  |  |  |  |  |  |  |  |  |  |
| Agricultural Derivatives |  |  |  |  |  |  |  |  |  |  |  |
| Agricultural Derivatives - Nutron |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Click Once - Auto FE Upgrade |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test Manual FE download |
| InfoMax EOD Files (AD product) |  | - | - |  | - |  | Ensure interface included in new Config file |  |  |  | Test File download |
| ISDN Dial-Up (Modem) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| ISDN Dial-Up (Router) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Agricultural Derivatives Test Environment |  |  |  |  |  |  |  |  |  |  |  |
| Agricultural Derivatives - Nutron |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Click Once - Auto FE Upgrade |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test Manual FE download |
| InfoMax EOD Files (AD product) |  | - | - |  | - |  | Ensure interface included in new Config file |  |  |  | Test File download |
| ISDN Dial-Up (Modem) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| ISDN Dial-Up (Router) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Yield-X |  |  |  |  |  |  |  |  |  |  |  |
| Production |  |  |  |  |  |  |  |  |  |  |  |
| Spots |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Derivatives |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Download FE Software |  | - | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test Manual FE download |
| Datafeed (Data vendors only) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test File download |
| Production DR |  |  |  |  |  |  |  |  |  |  |  |
| Spots |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Derivatives |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test Manual FE download |
| Download FE Software |  | - | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test File download |
| Datafeed (Data vendors only) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Yield-X Test Environment |  |  |  |  |  |  |  |  |  |  |  |
| Spots |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Derivatives |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Login test sufficient |
| Download FE Software |  | - | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test Manual FE download |
| Datafeed (Data vendors only) |  | Connect to New IP Addr Same Port | New route per interface to new IP Range |  | New route per interface to new IP Range |  | Apply new interface Config changes |  |  |  | Test File download |
| Other Services |  |  |  |  |  |  |  |  |  |  |  |
| Broker Deal Accounting (BDA) |  | - | - |  | - |  | Ensure interface included in new Config file | - |  |  | If you receive the JSE Logon Logo connection Proven |
| EOD Dissemination Download (Port 21) |  | - | - |  | - |  | Ensure interface included in new Config file | - |  |  | Connect to JSE Mainframe and Upload / Download Applicable File |
| FTP Upload for Alloc/Jnls (Port 20) |  | - | - |  | - |  | Ensure interface included in new Config file | - |  |  | Download any EOD Dissemination file |
| ERD / Control DPC |  | - | - |  | - |  | Ensure interface included in new Config file | - |  |  | Login to ERD and download a report |
| Print to Customer Remote Printer |  | - | - |  | - |  | Ensure interface included in new Config file | - |  |  |  |
| Print to Customer Remote CHEQUE Printer |  | - | - |  | - |  | Ensure interface included in new Config file | - |  |  |  |
| Trade Monitoring System (TMS) |  | - | - |  | - |  | Ensure interface included in new Config file | - |  |  | Login test sufficient |
| WEB Capad Submissions |  | - | - |  | - |  | - | - |  |  |  |
| Credit Inform |  |  |  |  |  |  | Ensure interface included in new Config file |  |  |  | Login test sufficient |

# 4. Equities Enablement’s

| Equities |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| JSE TradElect - SII | Software | USAP | Local IP Address | Enablement ID | LSE/MTN NAT IP | Primary/Backup |
| Infowiz - IRI | Software | USAP | Local IP Address | Enablement ID | LSE/MTN NAT IP | Primary/Backup |
| Talx - ASP |  |  |  |  |  |  |

# 5. Equity IP Fact Sheet:

| Equities |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equities | Current IP |  |  |  |  | New IP |  |  |
| Production | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Message retrieval service IRI | 196.26.125.221 | tcp |  | 62200 |  | 196.216.152.221 | 62200 | 2 |
| Secure interactive SII | 196.26.125.220 | tcp |  | 61100 |  | 196.216.152.220 | 61100 | 2 |
| CDS |  |  |  |  |  |  |  |  |
| IRI CDS | 10.5.7.110 | tcp |  | 52200 |  | 196.216.152.110 | 52200 | 2 |
| SII CDS | 10.5.7.100 | tcp |  | 51100 |  | 196.216.152.100 | 51100 | 2 |
| Conformance |  |  |  |  |  |  |  |  |
| IRI Conformance | 10.5.6.110 | tcp |  | 52800 |  | 196.216.153.110 | 52800 | 2 |
| SII Conformance | 10.5.6.100 | tcp |  | 51800 |  | 196.216.153.100 | 51800 | 2 |
| TALX - ASP Users | Current IP |  |  |  |  | New IP |  |  |
| ASP Server | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Titan | 196.26.125.80 | tcp |  | 21910 |  | 196.216.152.80 | 21910 | 2 |
| Hyperion | 196.26.125.82 | tcp |  | 21910 |  | 196.216.152.82 | 21910 | 2 |
| titansle1 | 196.26.125.211 | tcp |  |  |  | 196.216.152.211 |  |  |
| titansle2 | 196.26.125.212 | tcp |  |  |  | 196.216.152.212 |  |  |
| titansle3 | 196.26.125.213 | tcp |  |  |  | 196.216.152.213 |  |  |
| titansle4 | 196.26.125.214 | tcp |  |  |  | 196.216.152.214 |  |  |
| hypersle1 | 196.26.125.215 | tcp |  |  |  | 196.216.152.215 |  |  |
| hypersle2 | 196.26.125.216 | tcp |  |  |  | 196.216.152.216 |  |  |
| hypersle3 | 196.26.125.217 | tcp |  |  |  | 196.216.152.217 |  |  |
| hypersle4 | 196.26.125.218 | tcp |  |  |  | 196.216.152.218 |  |  |
| Multicast | Current IP |  |  |  |  | New IP |  |  |
|  | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| RP Address CTS and Prod | 196.26.125.105 | pim |  | pim |  | 196.216.152.105 | pim | n/a |
| Multicast Source Prod | 196.26.125.65 | udp |  | Prod BDG's |  | 196.216.152.65 | Prod BDG's | n/a |
| Multicast Source CTS | 196.26.123.91 | UDP |  | CDS/Conf BDG's |  | 196.216.152.99 | CDS/Conf BDG's | n/a |
| BDG Multicast Address range | Current IP (IP Remains the same) |  |  |  |  |  |  |  |
| Production |  | Protocol |  | Group Name |  | Multicast IP | Port number | FWSM |
| JSE Start of Day Public Order Book |  | udp |  | J01 |  | 239.5.5.1 | 49301 | 2 |
| Market Reference data - FULL download |  | udp |  | B10 |  | 239.1.1.10 | 63010 | 2 |
| African Market Rules FULL Download |  | udp |  | J02 |  | 239.5.5.2 | 49302 | 2 |
| African Market Reference Data FULL Download |  | udp |  | J03 |  | 239.5.5.3 | 49303 | 2 |
| Market Reference data – CHANGES |  | udp |  | B11 |  | 239.1.1.11 | 63011 | 2 |
| African Market Rules –CHANGES |  | udp |  | J10 |  | 239.5.5.10 | 49310 | 2 |
| African Market Reference Data -CHANGES |  | udp |  | J11 |  | 239.5.5.11 | 49311 | 2 |
| JSE Full Order Book Level 2 |  | udp |  | J20 |  | 239.5.5.20 | 49320 | 2 |
| JSE Best Bid and Ask Level 1 |  | udp |  | J21 |  | 239.5.5.21 | 49321 | 2 |
| JSE Trades |  | udp |  | J22 |  | 239.5.5.22 | 49322 | 2 |
| JSE Market Status Indicators |  | udp |  | J23 |  | 239.5.5.23 | 49323 | 2 |
| JSE VWAP |  | udp |  | J24 |  | 239.5.5.24 | 49324 | 2 |
| JSE Cumulative number and volume of trades |  | udp |  | J25 |  | 239.5.5.25 | 49325 | 2 |
| JSE News Service SENS |  | udp |  | J50 |  | 239.5.5.50 | 49350 | 2 |
| FTSE/JSE Africa Index Series |  | udp |  | J51 |  | 239.5.5.51 | 49351 | 2 |
| NSX Start of Day Public Order Book |  | udp |  | N01 |  | 239.5.5.81 | 49381 | 2 |
| NSX Market Data and News (SENS) |  | udp |  | N02 |  | 239.5.5.82 | 49382 | 2 |
| NSX Trades |  | udp |  | N03 |  | 239.5.5.83 | 49383 | 2 |
| FTSE/JSE Africa Index Series for NSX |  | udp |  | N04 |  | 239.5.5.84 | 49384 | 2 |
| CDS |  | Protocol |  | Group Name |  | Multicast IP | Port number | FWSM |
| CDS-B10 |  | udp |  | B10 |  | 239.199.10.10 | 55310 | 2 |
| CDS-B11 |  | udp |  | B11 |  | 239.199.10.11 | 55311 | 2 |
| CDS-J01 |  | udp |  | J01 |  | 239.199.15.1 | 55501 | 2 |
| CDS-J02 |  | udp |  | J02 |  | 239.199.15.2 | 55502 | 2 |
| CDS-J03 |  | udp |  | J03 |  | 239.199.15.3 | 55503 | 2 |
| CDS-J10 |  | udp |  | J10 |  | 239.199.15.10 | 55510 | 2 |
| CDS-J11 |  | udp |  | J11 |  | 239.199.15.11 | 55511 | 2 |
| CDS-J20 |  | udp |  | J20 |  | 239.199.15.20 | 55520 | 2 |
| CDS-J21 |  | udp |  | J21 |  | 239.199.15.21 | 55521 | 2 |
| CDS-J22 |  | udp |  | J22 |  | 239.199.15.22 | 55522 | 2 |
| CDS-J23 |  | udp |  | J23 |  | 239.199.15.23 | 55523 | 2 |
| CDS-J24 |  | udp |  | J24 |  | 239.199.15.24 | 55524 | 2 |
| CDS-J25 |  | udp |  | J25 |  | 239.199.15.25 | 55525 | 2 |
| CDS-J50 |  | udp |  | J50 |  | 239.199.15.50 | 55550 | 2 |
| CDS-J51 |  | udp |  | J51 |  | 239.199.15.51 | 55551 | 2 |
| CDS-N01 |  | udp |  | N01 |  | 239.199.15.81 | 55581 | 2 |
| CDS-N02 |  | udp |  | N02 |  | 239.199.15.82 | 55582 | 2 |
| CDS-N03 |  | udp |  | N03 |  | 239.199.15.83 | 55583 | 2 |
| CDS-N04 |  | udp |  | N04 |  | 239.199.15.84 | 55584 | 2 |
| Conformance |  | Protocol |  | Group Name |  | Multicast IP | Port number | FWSM |
| CONF-B10 |  | udp |  | B10 |  | 239.188.10.10 | 58310 | 2 |
| CONF-B11 |  | udp |  | B11 |  | 239.188.10.11 | 58311 | 2 |
| CONF-J01 |  | udp |  | J01 |  | 239.188.15.1 | 58501 | 2 |
| CONF-J02 |  | udp |  | J02 |  | 239.188.15.2 | 58502 | 2 |
| CONF-J03 |  | udp |  | J03 |  | 239.188.15.3 | 58503 | 2 |
| CONF-J10 |  | udp |  | J10 |  | 239.188.15.10 | 58510 | 2 |
| CONF-J11 |  | udp |  | J11 |  | 239.188.15.11 | 58511 | 2 |
| CONF-J20 |  | udp |  | J20 |  | 239.188.15.20 | 58520 | 2 |
| CONF-J21 |  | udp |  | J21 |  | 239.188.15.21 | 58521 | 2 |
| CONF-J22 |  | udp |  | J22 |  | 239.188.15.22 | 58522 | 2 |
| CONF-J23 |  | udp |  | J23 |  | 239.188.15.23 | 58523 | 2 |
| CONF-J24 |  | udp |  | J24 |  | 239.188.15.24 | 58524 | 2 |
| CONF-J25 |  | udp |  | J25 |  | 239.188.15.25 | 58525 | 2 |
| CONF-J50 |  | udp |  | J50 |  | 239.188.15.50 | 58550 | 2 |
| CONF-J51 |  | udp |  | J51 |  | 239.188.15.51 | 58551 | 2 |
| CONF-N01 |  | udp |  | N01 |  | 239.188.15.81 | 58581 | 2 |
| CONF-N02 |  | udp |  | N02 |  | 239.188.15.82 | 58582 | 2 |
| CONF-N03 |  | udp |  | N03 |  | 239.188.15.83 | 58583 | 2 |
| CONF-N04 |  | udp |  | N04 |  | 239.188.15.84 | 58584 | 2 |

# 6. Equity Derivatives IP Fact Sheet:

| Equity Derivatives | Current IP |  |  |  |  | New IP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Production | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Equity Derivatives - Nutron | 10.3.5.131 | tcp |  | 2003 |  | 196.216.152.131 | 2003 | 1 |
| Click Once - FE SW Downloads | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410/4411 | 1 |
| InfoMax EOD File (ED Product) | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410 | 1 |
| ISDN Dial-Up (Modem) | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410 | 1 |
| ISDN Dial-Up (Router) | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410 | 1 |
| Test | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Equity Derivatives - Nutron | 10.3.13.191 | tcp |  | 2003 |  | 196.216.152.191 | 2003 | 1 |
| Click Once - FE SW Downloads | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |
| InfoMax EOD File (ED Product) | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |
| ISDN Dial-Up (Modem) | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |
| ISDN Dial-Up (Router) | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |

# 7. Agri Derivatives IP Fact Sheet:

| Agricultural Derivatives | Current IP |  |  |  |  | New IP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Production | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Agri Derivatives - Nutron | 10.3.5.131 | tcp |  | 2210 |  | 196.216.152.131 | 2210 | 1 |
| Click Once - FE SW Downloads | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410/4411 | 1 |
| InfoMax EOD File (AD Product) | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410 | 1 |
| ISDN Dial-Up (Modem) | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410 | 1 |
| ISDN Dial-Up (Router) | 10.3.5.131 | tcp |  | 4410 |  | 196.216.152.131 | 4410 | 1 |
| Test | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Agri Derivatives - Nutron | 10.3.13.191 | tcp |  | 2210 |  | 196.216.152.191 | 2210 | 1 |
| Click Once - FE SW Downloads | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |
| InfoMax EOD File (AD Product) | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |
| ISDN Dial-Up (Modem) | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |
| ISDN Dial-Up (Router) | 10.3.13.191 | tcp |  | 4410 |  | 196.216.152.191 | 4410 | 1 |

# 8. Yield-X  IP Fact Sheet:

| Yield-X |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yield-X | Current IP |  |  |  |  | New IP |  |  |
| Production | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Spots | 10.3.4.44 | tcp |  | 2004 |  | 196.216.152.44 | 2004 | 1 |
| Derivatives | 10.3.4.44 | tcp |  | 2003 |  | 196.216.152.44 | 2003 | 1 |
| Downloads | 10.3.4.44 | tcp |  | 50000 |  | 196.216.152.44 | 50000 | 1 |
| Datafeed | 10.3.4.50 | tcp |  | 11424 |  | 196.216.152.50 | 11424 | 1 |
| Test | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Spots | 10.3.12.41 | tcp |  | 2004 |  | 196.216.152.41 | 2004 | 1 |
| Derivatives | 10.3.12.41 | tcp |  | 2003 |  | 196.216.152.41 | 2003 | 1 |
| Downloads | 10.3.12.41 | tcp |  | 50000 |  | 196.216.152.41 | 50000 | 1 |
| Datafeed | 10.3.12.50 | tcp |  | 11424 |  | 196.216.152.51 | 11424 | 1 |
| DR | IP | Protocol |  | Port Number |  | New NAT | Port Number | FWSM |
| Spots | 10.3.4.40 | tcp |  | 2004 |  | 196.216.152.40 | 2004 | 1 |
| Derivatives | 10.3.4.40 | tcp |  | 2003 |  | 196.216.152.40 | 2003 | 1 |
| Downloads | 10.3.4.40 | tcp |  | 50000 |  | 196.216.152.40 | 50000 | 1 |
| Datafeed | 10.3.4.52 | tcp |  | 11424 |  | 196.216.152.52 | 11424 | 1 |

# 8. Other IP Fact Sheet:

| Other |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BDA | Current IP (IP Remains the same) |  |  |  |  |  |  |  |
|  |  | Protocol |  | Port Number |  | IP | Port Number | FWSM |
| TN3270 |  | tcp |  | 23 |  | 196.26.120.0/25 | 23 | 1 |
| TN3270 |  | tcp |  | 23 |  | 196.26.122.0/24 | 23 | 1 |
| TMS | Current IP (IP Remains the same) |  |  |  |  |  |  |  |
|  |  | Protocol |  | Port Number |  | IP | Port Number | FWSM |
| TMS Client |  | tcp |  | 3007 |  | 196.26.120.242 | 3007 | 1 |
| FTP | Current IP (IP Remains the same) |  |  |  |  |  |  |  |
|  |  | Protocol |  | Port Number |  | IP | Port Number | FWSM |
| FTP-DATA |  | tcp |  | 20 |  | 196.26.120.242 | 20 | 1 |
| FTP |  | tcp |  | 21 |  | 196.26.120.242 | 21 | 1 |
| ERD |  | tcp |  | 2717 |  | 196.26.120.242 | 2717 | 1 |
| Remote Printing |  | tcp |  | 23 |  | 196.26.120.0/25 | 23 | 1 |
| Remote Cheque Printing |  | tcp |  | 23 |  | 196.26.120.0/25 | 23 | 1 |
| NOTE: If a PASSIVE FTP method is used, e.g. from a command prompt, only TCP port 21 is used. If a program requires ACTIVE FTP, then 20 and 21 are used, Port 21 for LOGIN, Port 20 for the data transfer in ACTIVE FTP mode. |  |  |  |  |  |  |  |  |

# 9. Cutover Contact List

| JSE Contacts | Department | Land Line | Mobile |
| --- | --- | --- | --- |
| Johnny Smith | ITD – Project Manager | 011 520 7713 | 0824918225 |
| Bennie Thomas | Nettech | 011 520 7433 |  |
| Petrus Joubert | Nettech | 011 520 7095 |  |
| Andre Botes | Nettech | 011 520 7437 |  |
| Catherine de Klerk | CSS | 011 520 7874 |  |
| Silvano Pizzocaro | CSS | 011 520 7777 |  |
| Peshen Reddy | CSS | 011 520 7777 |  |
| Member Contacts |  |  |  |
| Member Escalation Contacts |  |  |  |
| Vendor Contacts |  |  |  |