CONFIDENTIAL

NoteA1145B

User Connectivity to the new JSE

Derivatives and Yield-X Trading

Systems

Version 2.13

November 2009

Produced by:	JSE Application Management

Reviewed by:	JSE Application Management

Version:	Version 2.13

Date:		23 November 2009

This document is confidential and contains property belonging to the JSE Limited (the JSE).  Neither this document nor its contents may be disclosed to a third party, nor may it be copied, without the JSE’s prior written consent.

CONFIDENTIAL

The JSE endeavours to ensure that the data and other material in this document are correct and complete but does not accept any liability for error herein or omission here from.  The development of JSE products and services is continuous and published information may not be up to date.  It is important that you check the current position with the JSE before acting on any information in this document.

| TABLE OF CONTENTS |  |
| --- | --- |

1.	References	4

2.	Version Control	4

3.    Introduction	4

4.    Glossary of terms	4

5.     User Connectivity	5

5.1 Connectivity to the JSE Network	5

5.2 Configuring Router for IP	5

5.3 Firewall / Router Configuration	9

5.4 Routing	9

5.5 Bandwidth Requirements	9

5.6 Client workstation minimum requirements	10

5.7 Information Subscriber requirements for the new Derivatives Trading System	11

5.8 Market times for the Production and External Test Environments............................11

6.     Contact List for Help or queries	11

# References

| Document* | Author | Version | Issue Date |
| --- | --- | --- | --- |

# Version Control

| Version | Author | Date | Reason for Changes |
| --- | --- | --- | --- |
| Version 2.0 | Matthias Kempgen | 12 February 2007 | Bandwidth requirements, Front End section, IP addreses |
| Version 2.1 | Matthias Kempgen | 13 February 2007 | Added NAS request process |
| Version 2.2 | Matthias Kempgen | 13 February 2007 | Glossary updated |
| Version 2.3 | Matthias Kempgen | 01 August 2007 | Added Production IP addresses and port numbers |
| Version 2.4 | Matthias Kempgen | 16 October 2007 | Added Windows VISTA as a supported Operating System |
| Version 2.5 | Matthias Kempgen | 01 March 2008 | Updated Bandwidth requirements and PC specifications Added backup link note under ‘Connectivity to the JSE network |
| Version 2.6 | Matthias Kempgen | 11 June 2008 | Link to download MSXML4 |
| Version 2.7 | Matthias Kempgen | 31 July 2008 | Added Prod and QA availability and market times |
| Version 2.8 | Matthias Kempgen | 14 October 2008 | Amended open order period ahead of Agris go-live in section 5.10 |
| Version 2.9 | Matthias Kempgen | 04 May 2009 | Added IP addresses for NUTRON post JSE router cutover project |
| Version 2.10 | Matthias Kempgen | 20 June 2009 | Addition of external test environments for the new Yield-X markets |
| Version 2.11 | Matthias Kempgen | 26 August 2009 | Addition of Yield-X production environment details |
| Version 2.12 | Matthias Kempgen | 12 November 2009 | Addition of production Yield-X Market times |
| Version 2.13 | Matthias Kempgen | 23 November 2009 | Addition of new EDM and APD external test environment IP addresses |

# 3. Introduction

- The purpose of this document is to illustrate the connectivity options available to customers (Trading Members, Clearing Members and Information Subscribers) who will access the new JSE trading systems for Commodity Derivatives, Equity Derivatives and Yield-X.

- The JSE reserves the right to change IP addresses and port numbers of the new trading system environments but will communicate any such changes.

- Information Subscribers – please refer specifically to section 5.9 for information on infrastructure requirements for connection to the new systems via the API as well as all other sections of the document

# 4.  Glossary of terms

| IP | Internet Protocol |
| --- | --- |
| IPX | Novell Netware protocol (Internetwork Packet Exchange) |
| ISDN | Integrated Services Digital Network |
| JSE | JSE Ltd |
| NUTRON | NU TRading ONline |
| TCP | Transmission Control Protocol |
| Users | Recipients of JSE derivatives trading and information services |
| NAS | Network Access Service |

# 5. User Connectivity

## 5.1 Connectivity to the JSE Network

Currently members connect to the JSE systems using dial-up, ISDN or leased lines, with routers configured for TCP/IP.

To access the new system various options are available:

- Users can connect via existing leased lines. Users need to consider bandwidth utilisation on these circuits and impact to their current trading activities, and may as a result need to consider bandwidth upgrades to existing circuits. Routers must be configured for IP (see section 3.2 below).

- Members can elect to install new leased lines to facilitate connectivity to the new trading system.

- MTN Managed Network option – this option will enable users to connect to the JSE via the MTN MPLS network offering cost advantages and introducing connectivity options like leased line connectivity to a local MTN POP.

- ISDN Router dialup to the Commodities, Equity Derivatives and Yield-X external user test environment and production environments is also available

Additional requirements for connectivity to the new trading system are:

- Private (as allocated by the JSE) class IP address space or public class IP address space

- Cisco router (recommended)

- In light of the soon to be released connectivity requirements market participants are reminded that a backup link to the JSE will become a mandatory requirement.

If the member chooses to use their own public class IP address space this needs to be communicated to the JSE via the Derivative_Info@jse.co.za or Yield-X_Info@jse.co.za.

## 5.2 Configuring Router for IP

The member’s network and interface cards need to be enabled for TCP / IP connectivity. Sample router and switch configurations for TCP / IP networks can be obtained from www.cisco.com or member’s can contact their IT support provider.

For the JSE to configure members on TCP / IP, information is required and should be submitted to derivative_info@jse.co.za or yield-x_info@jse.co.za as a request for configuration.

Should a member wish to connect via the MTN managed network solution members will be assisted with the setup.

## 5.3 Firewall / Router Configuration

- The following information is required to update firewall/router configurations if installed:

- Pre-Router Cutover IP addressing – ie clients connected via old Router infrastructure

- Test environment

- Test Equity Derivatives Market and Commodity Derivatives:

- Proxy host address:	 	                                                               10.6.224.30

TCP port (Trading Engine - Equity Derivatives): 		                                     2003

TCP port (Trading Engine - Commodity Derivatives): 	                                     2210

- TCP Port (Trading Engine – Yield-X Market)					 2269

- TCP port (Click Once - Equity Derivatives and Commodity Derivatives):	             4410

- TCP Port (Click Once – Yield-X Market)                                                               4469

- Production environment

- Production Equity Derivatives Market and Commodity Derivatives:

Proxy host address: 		                                                                10.3.5.131

TCP port (Trading Engine - Equity Derivatives): 	                                                2003

TCP port (Trading Engine - Commodity Derivatives): 	                                    2210

TCP port (Click Once - Equity Derivatives and Commodity Derivatives):	4410 or 4411

- Production Yield-X Spot and Derivatives Markets

Proxy host address: 		                                                                10.3.5.131

- TCP Port (Trading Engine – Yield-X Market)					2269

- TCP Port (Click Once – Yield-X Market)                                                              4469

- Post-Router Cutover IP addressing – ie clients connecting via new router infrastrcuture:

- Test environment

- Test Equity Derivatives Market and Commodity Derivatives:

- Proxy host address:	 	                                                          196.216.152.30

TCP port (Trading Engine - Equity Derivatives): 		                                     2003

TCP port (Trading Engine - Commodity Derivatives): 	                                     2210

- TCP Port (Trading Engine – Yield-X Market)					 2269

- TCP port (Click Once - Equity Derivatives and Commodity Derivatives):	 4410 or 4411

- TCP Port (Click Once – Yield-X Market)                                                                4469

- Production environment

- Production Equity Derivatives Market and Commodity Derivatives:

Proxy host address: 		                                                       196.216.152.131

TCP port (Trading Engine - Equity Derivatives): 	                                                2003

TCP port (Trading Engine - Commodity Derivatives): 	                                    2210

TCP port (Click Once - Equity Derivatives and Commodity Derivatives):	4410 or 4411

- Production Yield-X Spot and Derivatives Markets

Proxy host address: 		                                                            196.216.152.131

- TCP Port (Trading Engine – Yield-X Market)					2269

- TCP Port (Click Once – Yield-X Market)                                                              4469

## 5.4 Routing

- Customers must use static routing to route data across the interface between the Customer Network and the JSE Network.

- The following IP network configurations are required on the routers to facilitate connectivity:

- Production and Test environments:

- IP route 196.216.152.0 255.255.254.0

## 5.5 Bandwidth Requirements

- Bandwidth Requirements for the new Derivatives Trading System Front End:

  - APD – 25k on average per user during a trading day (based on current production activity                          and will vary depending on the number of users per participating customer)

- Customers are requested to cater for the possibility of temporary spikes due to increased market activity

  - EDM – 25k on average per user during a trading day (based on current production activity                          and will vary depending on the number of users per participating customer)

    - Users can configure their front end to only ‘listen’ for updates on the instruments they are interested in viewing.

  - Yield-X – 35k on average per user during a trading day (based on current production activity                          and will vary depending on the number of users per participating customer)

    - Users can configure their front end to only ‘listen’ for updates on the instruments they are interested in viewing.

    - Please note that users need to cater for sufficient bandwidth in addition to this if they wish to use any algorithmic or autoquoting software/functionality (including the JSE’s own autoquote engine). This will also apply to users who wish to view all active instruments with autoquoting activity.

    - In addition JSE autoquote enabled users are able to view the underlying prices for all autoquote enabled instruments – this functionality requires at least 10k per user depending on the number of linked instruments

    - Customers are requested to cater for the possibility of temporary spikes due to increased market activity

- Bandwidth Requirements for Information Subscribers:

  - The JSE recommends 64k per ‘listener’ per market when listening to all instruments available.

- EOD file downloads:

- Specifications on these files are being finalised, however the JSE estimates that the complete set of available files will not exceed 2MB per day per market if all files are subscribed to.

## 5.6 Client workstation minimum requirements

The new NUTRON front end application should be installed on a PC with the following recommended specifications:

Recommended:

P4 2.8 GHz or equivalent processor

512 MB RAM

Windows XP sp2

Minimum 2GB available disk space

Network Interface Card

OR

P4 2.8 GHz or equivalent processor

1024 MB RAM

Windows VISTA

Minimum 2GB available disk space

Network Interface Card

PLEASE NOTE:  The file downloads to HTML and XML from the front end require the MSXML4 pack to be installed on the workstation. This can be downloaded from:

http://www.microsoft.com/downloads/details.aspx?familyid=3144b72b-b4f2-46da-b4b6-c5d7485f2b42&displaylang=en

Users are advised that these base requirements do not cater for the trading front end in combination with other applications  (e.g. e- mail, pricing tools, etcetera). Each user is required to do sufficient testing to ensure their respective workstations are up to specification as required.

## 5.7 Information Subscriber requirements

Information Subscribers will make use of the same API as the trading members– the API document for the new Commodities, Equity Derivatives and Yield-X systems (which is available separately) specifically indicates messages to be used by Information Subscribers to connect and access public information.

Please note that the new Commodities, Equity Derivatives and Yield-X API is based on the TCP/IP communications protocol.

## 5.8 System availability and market times for the Production and External Test Environments

  - External Test Environment Times

Commodity Derivatives (all times South African Standard Time)

| Time | State of system | Comments |
| --- | --- | --- |
| 08h30 – 09h00 | Download only period |  |
| 09h15 – 09h30 | Open Order period for all instruments |  |
| 09h30 – 23h30 | Market open | External Test environment is unsupported between 17h00 – 8h30 |
| 23h30 – 23h55 | Market is in Admin period |  |
| 23h55 – 05h00 | Download only period |  |
| 05h00 – 08h30 | Market offline |  |

Equity Derivative Market (all times South African Standard Time)

| Time | State of system | Comments |
| --- | --- | --- |
| 08h30 – 09h00 | Download only period |  |
| 09h15 – 09h30 | ALSI contract in opening auction |  |
| 09h30 – 23h30 | Market open | External Test environment is unsupported between 17h00 – 8h30 |
| 23h30 – 23h55 | Market is in Admin period |  |
| 23h55 – 05h00 | Download only period |  |
| 05h00 – 08h30 | Market offline |  |

Yield-X

| Time | State of system | Comments |
| --- | --- | --- |
| 08h30 – 09h00 | Download only period |  |
| 09h30 – 23h30 | Market open – all instruments in continuous trading | External Test environment is unsupported between 17h00 – 8h30 |
| 23h30 – 23h55 | Market is in Admin period |  |
| 23h55 – 05h00 | Download only period |  |
| 05h00 – 08h30 | Market offline |  |

  - Production Environment Times

Commodity Derivatives (all times South African Standard Time)

| Time | State of system | Comments |
| --- | --- | --- |
| 05h00 – 08h50 | Download only period |  |
| 08h50 – 8h59 | Open Order period for all instruments |  |
| 09h00 – 12h00 | Market open |  |
| 12h00 – 12h45 | Market is in Admin period |  |
| 12h45 – 03h00 | Download only period |  |
| 03h00 – 05h00 | Market offline |  |

Equity Derivatives Market (all times South African Standard Time)

| Time | State of system | Comments |
| --- | --- | --- |
| 05h00 – 08h25 | Download only period |  |
| 08h25 – 08h30 | ALSI contract in opening auction |  |
| 08h30 – 17h30 | Market open |  |
| 17h30 – 18h00 | Market is in Admin period |  |
| 18h00 – 03h00 | Download only period |  |
| 03h00 – 05h00 | Market offline |  |

Yield-X

| Time | State of system | Comments |
| --- | --- | --- |
| 05h00 – 08h00 | Download only period |  |
| 08h00 | All instruments EXCEPT currency futures in continuous trading |  |
| 09h00 | Currency Futures Continuous Trading |  |
| 17h00 – 17h15 | Market is in Admin period |  |
| 17h15 – 03h00 | Download only period |  |
| 03h00 – 05h00 | Market offline |  |

# 6. Contact List for help or queries

- Software or Technical Queries:

- The JSE Trading Division can be contacted via the following e-mail address:

- Derivative_Info@jse.co.za